# Documentation of APS model file format (xml format)

Version: 1.1

An example of a model file is enclosed. It shows the various elements that is necessary to specify a model for one reservoir zone. The model is specified within the top-level keyword:

<APSModel version=”1.1”>

## Common project data

Name of RMS project having the workflow this APS model is a part of

Keyword: RMSProjectName

Parent keyword: APSModel

Description: Define name of RMS project where this modelfile originally came from.

Keyword: GridModelName

Parent keyword: APSModel

Description: Define name of grid model this APS model belongs to.

Keyword: ZoneParamName

Parent keyword: APSModel

Description: Define name of zone parameter for the grid model.

Keyword: ResultFaciesParamName

Parent keyword: APSModel

Description: Define name of result 3D facies parameter.

Keyword: JobSettings

Parent keyword: APSModel

Description: Define settings common for the APS job. Is related to parameters in the JobSettings window.

Example:  <JobSettings> … </JobSettings >

Sub keywords:

- FmuSettings, RunSettings, TransformationSettings, LogSetting

Keyword: FmuSettings

Parent keyword: JobSettings

Description: Specify FMU related functionality for APS.

Example:  <FmuSettings> … </FmuSettings>

Sub keywords:

- FmuMode, UpdateGRF, UseResidualFields, UseNonStandardFMU, ExportConfigFiles

Keyword: FmuMode

Parent keyword: FmuSettings

Example:  <FmuMode> OFF </FmuMode>

Description: Specify in which mode APS is running. The alternatives are OFF, NOFIELDS, FIELDS.

- OFF means not read or update Fmu parameters

- NOFIELDS means read global_variables.yml file to get input model parameters

- FIELDS means read global_variables.yml file to get input model parameters and write simulated gaussian field parameters to file to be read by ERT or read such files after updated by ERT.

Keyword: UpdateGRF

Parent keyword: FmuSettings

Description: Specify ERTBOX grid, simulate or import field parameters, file format, extrapolation method for inactive values in ERTBOX grid.

Example:  <UpdateGRF>…</UpdateGRF>

Sub keywords:

- ErtBoxGrid, ExportErtBoxGrid, ExchangeMode, FileFormat, ExtrapolationMethod

Keyword: ErtBoxGrid

Parent keyword: UpdateGRF

Description: Specify ERTBOX grid name

Example:  <ErtBoxGrid> ertbox_Valysar </ErtBoxGrid>

Keyword: ExportErtBoxGrid

Parent keyword: UpdateGRF

Description: Specify if the ertbox grid is to be saved to file or not. Values are YES, NO.

Example:  <ExportErtBoxGrid> YES </ExportErtBoxGrid>

Keyword: ExchangeMode

Parent keyword: UpdateGRF

Description: Specify if gaussian field is to be simulated and written to file or imported from file.
Values are SIMULATE or AUTO.

Example:  <ExchangeMode> SIMULATE </ExchangeMode>

Keyword: FileFormat

Parent keyword: UpdateGRF

Description: Specify ROFF or GRDECL file as format for field parameters.

GRDECL file is now deprecated and will be removed.
Example:  <FileFormat> ROFF </FileFormat>

Keyword: ExtrapolationMethod

Parent keyword: UpdateGRF

Description: Specify extrapolation method for field parameters for inactive grid cells in ERTBOX grid.

Alternatives are:

- mean for using average value based on all values in the grid,

- extend which vertically extrapolate values using last defined value per column in the grid.

- repeat will copy the defined values of a column up or down to inactive cells by repeating
the values. Other alternatives are extend_layer_mean, repeat_layer_mean.

Example:  <ExtrapolationMethod> extend </ExtrapolationMethod>

Keyword: UseResidualFields

Parent keyword: FmuSettings

Description: Specify to only export/import the residual fields to/from ERT.

APS will add the trend if any before applying truncation rule. Values are YES, NO.
Example:  < UseResidualFields> NO </UseResidualFields>

Keyword: UseNonStandardFMU

Parent keyword: FmuSettings

Description: Enable use of additional configuration file to set non-standard filepaths for FMU projects.

Values are YES, NO.
Example:  < UseNonStandardFMU> NO </UseNonStandardFMU>

Keyword: ExportConfigFiles

Parent keyword: FmuSettings

Description: Turn on/off if model file for APS and additional FMU related configuration files

or templates for configuration files is to be exported from APS.

Values are YES, NO.
Example:  <ExportConfigFiles> NO </ExportConfigFiles>

Keyword: RunSettings

Parent keyword: JobSettings

Description: Specify tolerances for inaccuracies in normalization of facies probability cubes.

Example:  <RunSettings> … </RunSettings>

Sub keywords:

- MaxFractionNotNormalised, ToleranceLimitProbability

Keyword: MaxFractionNotNormalised

Parent keyword: RunSettings

Description: Specify max fraction of grid cells in probability cube having values outside tolerance limit.

Value between 0.0 and 1.0

Example:  <MaxFractionNotNormalised> 0.1 </MaxFractionNotNormalised>

Keyword: ToleranceLimitProbability

Parent keyword: RunSettings

Description: Specify tolerance limit for facies probabilities in input facies probability cubes.

Value between 0.0 and 1.0

Example:  <ToleranceLimitProbability> 0.1 </ ToleranceLimitProbability>

Keyword: TransformationSettings

Parent keyword: JobSettings

Description: Specify transformation method for gaussian fields to get values between 0 and 1 for the truncation map/cube. Values are EMPIRIC or 0 as one alternative, and CDF or 1 as the second alternative. EMPIRIC is always used if trends in GRF’s.

Example:  <TransformationSettings> 0 </TransformationSettings>

Keyword: LogSetting

Parent keyword: JobSettings

Description: Specify how much to print to screen as log output. Values are 0,1,2,3 with 0 as almost nothing and 3 as very much.

Example:  <LogSetting> 1 </LogSetting>

Global facies table:

Keyword: MainFaciesTable

Parent keyword: APSModel

Description: The global facies table is a table with all facies names that are used for all zones in the facies modelling of the specified grid model. Hence the APS model will in general use a subset of these facies, e.g. if the facies combinations vary from zone to zone or if some zones are not modelled by the APS method. The global facies table is a list of all facies names and their associated facies code that is used in the 3D facies realization parameters (where each grid cell is assigned a facies code).

Example:  <MainFaciesTable> … </MainFaciesTable>

Sub keywords:

- Facies

Keyword: Facies

Parent keyword: MainFaciesTable

Attribute: name

Description: Specify facies in the main or global facies table. The attribute is the name of the facies.

Example:  <Facies name="F1">..</Facies>

Sub keyword: Code

Keyword: Code

Parent keyword: Facies

Description: Specify a facies code for the specified facies. The facies code is a positive integer used in the facies realization. Each grid cell in the facies realization for a grid model is assigned a facies code from the main facies table.

Example:  <Code> 1 </Code>

## Zone specific model parameters

An APS model can be specified in all or a subset of all zones belonging to a grid model.

Keyword: ZoneModels

Parent keyword: APSModel

Sub-keywords:

- Zone

Keyword: Zone

Parent keyword: ZoneModels

Attribute: number and regionNumber

Description: Specify the model parameter for a zone. The attribute number is the zone number and optionally also region number.

Example: <Zone number="1"> … </Zone>

Example: <Zone number="1" regionNumber=”1”> … </Zone>

Sub-keywords:

- GridLayout

- UseConstProb

- SimBoxThickness

- FaciesProbForModel

- GaussField

- TruncationRule

Keyword: GridLayout

Parent keyword: Zone

Description: Specify the grid conformity of the geogrid model when using APS in FMU mode with simulation and exchange of gaussian fields with ERT. Possible values are:

- BaseConform

- TopConform

- Proportional

Keyword: UseConstProb

Parent keyword: Zone

Description: Integer number 0 or 1 where 0 means that the specified facies probabilities specified under keyword FaciesProbForModel are not constants, but a name of a 3D continuous parameter containing facies probability cube. The value 1 means that the specified facies probabilities are constants (the same value for all grid cells in the zone) and specified by float numbers for the probabilities.

Example: <UseConstProb> 1 </UseConstProb>

Keyword: SimBoxThickness

Parent keyword: Zone

Description: Float number > 0. This is the thickness of the simulation box used in RMS for the zone. This parameter must be consistent with the simulation box thickness in the RMS project for the zone in the grid model specified. It is used when calculating trends for Gaussian fields.

Example: <SimBoxThickness> 4.0 </SimBoxThickness>

Keyword: FaciesProbForModel

Parent keyword: Zone

Description: This keyword contains specification of list of facies to be used in the APS model and their facies probabilities.

Example:  <FaciesProbForModel> … </FaciesProbForModel>

Sub-keywords:

- Facies

Keyword: Facies

Parent keyword: FaciesProbForModel

Attribute: name

Description: It specify the name of the facies using the name attribute.

Example: <Facies name="F1"> … </Facies>

Sub-keywords:

- ProbCube

Keyword: ProbCube

Parent keyword: Facies

Description: This keyword is a child of the Facies keyword which is again a child of keyword FaciesProbForModel. It specifies either a float number or a text word containing the name of a 3D parameter for the facies probability for the facies it belongs to.

Example: <ProbCube> 0.2 </ProbCube>

Keyword: GaussField

Attribute: name

Parent keyword: Zone

Description: Specify name of Gaussian fields and their model parameters through the child keywords.

Example: <GaussField name="GF1"> … </GaussField>

Sub-keywords:

- Vario

- Trend

- RelStdDev

- SeedForPreview

Keyword: Vario

Attribute: name

Parent keyword: GaussField

Description: Specify variogram model parameters for Gaussian field through the child keywords.

The attribute “name” specifies the variogram type. The possible allowed values are:

- GENERAL_EXPONENTIAL

- EXPONENTIAL

- SPHERICAL

- GAUSSIAN

- MATERN32

- MATERN52

- MATERN72

Example: <Vario name="GENERAL_EXPONENTIAL"> … </Vario>

Sub-keywords:

- MainRange

- PerpRange

- VertRange

- AzimuthAngle

- DipAngle

- Power

Keyword: MainRange

Parent keyword: Vario

Description: Specify correlation length in main correlation direction. Allowed values are positive float numbers.

Example: <MainRange> 5000.0 </MainRange>

Keyword: PerpRange

Parent keyword: Vario

Description: Specify correlation length in perpendicular correlation direction. Allowed values are positive float numbers.

Example: <PerpRange> 5000.0 </PerpRange>

Keyword: VertRange

Parent keyword: Vario

Description: Specify correlation length in vertical correlation direction. Allowed values are positive float numbers.

Example: <VertRange> 2.0 </VertRange>

Keyword: AzimuthAngle

Parent keyword: Vario

Description: Specify azimuth angle for direction for the main correlation direction. Allowed values are float numbers from 0 to 360 (degrees).

Example: <AzimuthAngle> 2.0 </AzimuthAngle>

Keyword: Power

Parent keyword: Vario

Description: Specify exponent in general exponential variogram. Only relevant if general exponential variogram is specified as variogram type in the parent keyword. Allowed values are from 1.0 to 1.99.

Example: <Power> 1.8 </Power>

Keyword: Trend

Parent keyword: GaussField

Description: Specify a trend type to be used for the Gaussian field.

Example:  <Trend> … </Trend>

Sub-keywords: Alternatives: One of:

- Linear3D

- Elliptic3D

- EllipticCone3D

- Hyperbolic3D

- RMSParameter

- RMSTrendMap

Keyword Linear3D:

Parent Keyword: Trend

Description: Specify a Linear 3D trend type to be used for the Gaussian field.

Example:  <Linear3D> … </Linear3D>

Sub-keywords: azimuth, directionStacking, stackAngle

Keyword Elliptic3D:

Parent Keyword: Trend

Description: Specify Elliptic 3D trend type to be used for the Gaussian field.

Example:  < Elliptic3D> … </Elliptic3D>

Sub-keywords: azimuth, directionStacking, stackangle, curvature, origin_x, origin_y, origin_z_simbox, origintype

Keyword EllipticCone3D:

Parent Keyword: Trend

Description: Specify Elliptic Cone 3D trend type to be used for the Gaussian field.

Example:  <EllipticCone3D> … </EllipticCone3D>

Sub-keywords:  azimuth, directionStacking, stackAngle, migrationAngle, relativeSize,

curvature, origintype, origin_x, origin_y, origin_z_simbox

Keyword Hyperbolic3D:

Parent Keyword: Trend

Description: Specify a Hyperbolic 3D trend type to be used for the Gaussian field.

Example:  < Hyperbolic3D> … </ Hyperbolic3D>

Sub-keywords: azimuth, directionStacking, stackAngle, migrationAngle,

curvature, origintype, origin_x, origin_y, origin_z_simbox

Keyword RMSParameter:

Parent Keyword: Trend

Description: Specify an RMS parameter for trend type to be used for the Gaussian field.

Example:  < RMSParameter> … </RMSParameter >

Sub-keywords: TrendParamName

Keyword: azimuth

Parent keyword: Linear3D or Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: Direction (azimuth angle) for the linear trend. This is an angle between 0 and 360 degrees.

Example: <asimuth> 125.0 </asimuth>

Keyword: directionStacking

Parent keyword: Linear3D or Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: This parameter is 1 or -1 depending on stacking type (progradation or retrogradation)

Example: <directionStacking> 1 </directionStacking>

Keyword: stackAngle

Parent keyword: Linear3D or Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: This parameter is the dip angle of facies layers for facies belts that are created. Allowed values are in interval from 0 to 90 degrees.

Example: <stackAngle> 0.1 </stackAngle>

Keyword: curvature

Parent keyword: Elliptic3D or EllipticCone3D or Hyperbolic3D

Description: Curvature parameter for the trend (float value)

Example: <curvature> 0.1 </curvature>

Keyword: migrationAngle

Parent keyword: EllipticCone3D or Hyperbolic3D

Description: Parameter for how the trend moved sideways relative to azimuth direction with depth. (float value)

Example: < migrationAngle > 0.1 </migrationAngle >

Keyword: relativeSize

Parent keyword: EllipticCone3D

Description: Shape parameter for the cone. (float value)

Example: <relativeSize > 0.1 </relativeSize >

Keyword: origin_x

Parent keyword: Elliptic3D or EllipticCone3D or Hyperbolic3D

Description: Origin x coordinate for ellipse (float value)

Example: < origin_x> 0.1 </origin_x>

Keyword: origin_y

Parent keyword: Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: Origiin y coordinate for ellipse (float value)

Example: < origin_y> 0.1 </origin_y>

Keyword: origin_z_simbox

Parent keyword: Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: Origin z coordinate relative to simbox (float value)

Example: <origin_z_simbox> 0.1 </origin_z_simbox>

Keyword: origintype

Parent keyword: Eliptic3D or EllipticCone3D or Hyperbolic3D

Description: Enumeration with legal values Relative or Absolute

define if coordinates is relative to global coordinates or

relative to local simulation box where the position varies

from 0 to 1 in local x and y direction.

Example: < origintype > Relative </ origintype >

Keyword: RelStdDev

Parent keyword: GaussField

Description: Relative standard deviation. This parameter is used in the following calculations in the algorithm: The maximum and minimum value for the trend within the grid cells belonging to the zone is calculated. This difference is multiplied by the relative standard deviation to get the standard deviation. This standard deviation is multiplied by Gaussian fields with 0 expectation and unit variance to get the Gaussian noise to add to the trend.

Example:  <RelStdDev> 0.01 </RelStdDev>

Keyword: SeedForPreview

Parent keyword: GaussField

Description: Random seed used in preview simulation of the Gaussian field. Large integer number.

Example:  <SeedForPreview> 276787 </SeedForPreview>

Keyword: TruncationRule

Parent keyword: Zone

Description: Specify truncation rule. Has one of three alternative types of truncation rules.

Only one is specified. The child keyword has an attribute nGFields which specify the number of Gaussian fields the truncation rule is designed for. The allowed child keywords (type of truncation rules) are:

- Trunc2D_Cubic

- Trunc2D_Angle

- Trunc3D_Bayfill

Example:   <TruncationRule >

<Trunc3D_Bayfill nGFields="3">

…

</TruncationRule>

Keyword: Trunc2D_Cubic

Parent keyword: TruncationRule

Description: Specify truncation rule type ‘cubic’ using rectangular areas in truncation map/cube.

Sub-keywords:

- BackGroundModel (required)

- OverLayModel (optional)

Keyword: BackGroundModel (related to Trunc2D_Cubic)

Parent keyword: Trunc2D_Cubic

Description: Specify the 2D truncation rule based on two transformed gaussian fields. The facies

defined here is often called background facies and background for other facies types that can ‘overprint’ or erode into these facies. The eroding facies are specified as ‘overlay’ facies in the OverLayModel keyword.

Sub-keywords:

- AlphaFields

- L1

Keyword: AlphaFields

Parent keyword: BackGroundModel

Description: Define the names of the two transformed gaussian fields for axis 1 and 2 in truncation map.

Keyword: L1

Parent keyword: BackGroundModel

Description: Define the first level of the 3-level hierarchical truncation rule based on rectangular areas.

Attributes for L1 is ‘direction’ with value ‘V’ or ‘H’.

Sub-keywords:

- ProbFrac

- L2

Keyword: L2

Parent keyword: L1

Description: Define the second level of the 3-level hierarchical truncation rule based on rectangular areas.

Attributes for L2 is ‘direction’ with value ‘V’ or ‘H’.

Sub-keywords:

- ProbFrac

- L3

Keyword: L3

Parent keyword: L2

Description: Define thethird level of the 3-level hierarchical truncation rule based on rectangular areas.

Sub-keywords:

- ProbFrac

Keyword: ProbFrac

Parent keyword: L1 or L2 or L3

Description: Define facies name as attribute and probability fraction for this facies within the rectangular area/volume defined by this keyword.

Keyword: Trunc2D_Angle

Parent keyword: TruncationRule

Description: Specify truncation rule type non-orthogonal polygons. The attribute nGFields define the number of gaussian fields used for this truncation rule.

Sub-keywords:

- BackGroundModel (required)

- OverLayModel (optional)

Keyword: BackGroundModel (related to Trunc2D_Angle)

Parent keyword: Trunc2D_Angle

Description: Specify the 2D truncation rule based on two transformed gaussian fields. The facies

defined here is often called background facies and background for other facies types that can ‘overprint’ or erode into these facies. The eroding facies are specified as ‘overlay’ facies in the OverLayModel keyword.

Sub-keywords:

- AlphaFields

- Facies

- UseConstTruncParam (Not used, is set to 1)

Keyword: AlphaFields

Parent keyword: BackGroundModel

Description: Define the names of the two transformed gaussian fields for axis 1 and 2 in truncation map.

This the same as for the background model for cubic truncation rule.

Keyword: Facies

Parent keyword: BackGroundModel

Description: Define a polygon in the truncation map. Attribute is facies name.

Sub-keywords:

- Angle

- ProbFrac

Keyword: Angle

Parent keyword: Facies

Description: Define a line by an angle. The lines together will split the truncation map into polygons.

The polygon is associated with the facies specified with the attribute name.

The value is angle in degrees.

Keyword: ProbFrac

Parent keyword: Facies

Description: Define probability fraction for the facies probability assigned to the current polygon. If there are only one polygon associated to the facies for this polygon, the facies probability fraction is 1. If there exist multiple polygons with the same facies, the sum of the facies probability fraction over these polygons must be 1.

Keyword: OverLayModel

Parent keyword: Trunc2D_Cubic or Trunct2D_Angle

Description: Specify eroding facies that ‘overprints’ the background facies. Optional keyword.

Multiple groups of background facies are specified by the sub-keyword group.

Sub-keywords:

- Group

Keyword: Group

Parent keyword: OverLayModel

Description: Specify a group of background facies and which overlay facies is eroding into the background facies. Note that grouping of background facies must be non-overlapping. A background facies can be member of one group only or no group.

Sub-keywords:

- AlphaField

- Background

Keyword: Background

Parent keyword: Group

Description: Specify name of background facies belonging to the group.

Keyword: AlphaField

Parent keyword: Group

Description: Specify name of overlay facies with attribute name and define a new alpha coordinate axis (a new transformed gaussian field) for the truncation cube.

Sub-keywords:

- TruncationIntervalCenter

- ProbFrac

Keyword: TruncationIntervalCenter

Parent keyword: AlphaField

Description:  The new alpha axis has values from 0 to 1. The overlay facies is defined by truncating the alpha value along this axis. The truncation subdivides the interval from 0 to 1 into two intervals if TruncationIntervalCenter is set to 0 or 1 and split the interval into 2 or possibly 3 intervals if TruncationIntervalCenter is a value between 0 and 1.  The TruncationIntervalCenter defines the centerpoint of the truncation interval assocated with the overlay facies. If the new alpha value is within this interval, the overlay facies in ProbFrac keyword is assigned, if the alpha value is outside the interval around the center point, the background facies is kept. The size of the interval is defined by the facies probability and the fraction of that associated to this polygon.

Keyword: ProbFrac

Parent keyword: AlphaField

Description: Specify overlay facies probability fraction for current polygon. The polygon is here defined by name of overlay facies specified as the attribute name.

Keyword: Trunc3D_Bayfill

Parent keyword: TruncationRule

Description: Specify truncation rule type called Bayfill. The attribute nGFields must be 3.

Sub-keywords:

- BackGroundModel

Keyword: BackGroundModel

Parent keyword: Trunc3D_Bayfill

Description: Specify parameters for this truncation rule type.

Sub-keywords:

- AlphaFields

- UseConstTruncParam (Not used and should have value 1.)

- Floodplain

- Subbay

- WBF

- BHD

- Lagoon

- SF

- YSF

- SBHD

Keyword: AlphaFields

Parent keyword: BackGroundModel

Description: Specify 3 GRF names.

Keywords: Floodplain, Subbay, WBF, BHD, Lagoon

Parent keyword: BackGroundModel

Description: Specify facies names for the 5 different roles of facies.

Keyword: SF, YSF SBHD

Parent keyword: BackGroundModel

Description: Specify parameters with value between 0 and 1. The parameters define the shape of the polygons the 3D truncation cube is subdivided into.